BOTSWANA INTERNATIONAL UNIVERSITY OF SCIENCE & TECHNOLOGY

(SEMESTER 1, 2023/2024 FINAL EXAMINATIONS)

| FACULTY/DEPARTMENT: | FOE/MEIEM |
| --- | --- |
| MODULE CODE: | MINS 311 |
| MODULE TITLE: | Measurement and Instrumentation |
| DATE: | 26th September 2024 |
| TIME: | 08:00-09:00 (1 hours) plus 10 minutes reading time |
| VENUE(s): | Block 124/001-002 |
| EXAMINER(s): | Mrs Matebejana |

Instructions to Candidates:

- Answer all questions.

- Use only a Blue or Black pen for written answers and a pencil for all sketches/   drawings.

- Ensure that your question paper contains all pages.

- Only non-programmable calculators are allowed in this examination.

- Possession of any other material not prescribed in these instructions including mobile phones/smart watches is a punishable offence as per the BIUST Misconduct Regulation

DO NOT TURN OVER THE PAGE UNTIL YOU ARE TOLD TO DO SO

- What is the function of a variable conversion element in a measurement system?

To convert the output signal of the primary sensing element to another more suitable variable  while preserving the information content of the original  signal.

[2 marks]

- A current of 3.12A is flowing through a resistor of 53. 635ohms.Find the value of voltage drop across the resistor to the appropriate number of significant figures.

v=IR=3.12*53.635=167.3412

=167v

[2marks]

- What is a Transducer? [1mark]

- The cran is a British volume unit for freshly caught herrings: 1 cran=170.474 liters (L) of fish, about 750 herrings. Suppose that, to be cleared through customs in Saudi Arabia, a shipment of 1255 crans must be declared in terms of cubic covidos, where the covido is an Arabic unit of length: 1 covido=48.26 cm . What is the required declaration? [4marks]

- Explain the difference between accuracy and precision of measurement. On what factors does precision depend? How can the accuracy be improved?

[3marks]

  - Accuracy is the ability of an instrument to show the exact reading.

  - Always related to the extent of the wrong reading/non accuracy.

- Defined as the capability of an instrument to show the same reading when used each time (reproducibility of the instrument).

- Precision of a measurement depends on what the number of significant figures.

- The accuracy can be improved upon by better calibration of the instrument

- Discuss the three types of systematic errors giving suitable examples. Explain the measures taken to minimise these errors. [9marks]

Instrumental Errors

- Due to inherent shortcoming in the instrument.

Example:- If the spring used in permanent magnet  instrument has become weak then instrument will  always read high. Errors may caused because of  friction , hysteresis , or even gear backlash.

- Due to misuse of the instruments.

- Due to Loading effects of instruments.

Observational errors:-

-- Parallax, i.e. Apparent displacement when the line of vision is not normal to the scale.

-- Inaccurate estimate of average reading.

-- Wrong scale reading and wrong recording the data.

-- Incorrect conversion of units between consecutive reading.

- For the following given data, calculate

x1= 39.7; x2=40.1 ; x3=40.2; x4=39.6; x5=39.8

  - Arithmetic mean; [1mark]

  - Deviation of each value; [3marks]

  - Standard deviation [2marks]

- A voltmeter reading 70V on its 100V range and an ammeter reading 80mA on its 150mA range are used to determine the power dissipated in a resistor. Both these instruments are guaranteed to be accurate within ±1.5%  at full scale deflection. Determine the limiting error of the power.  [3marks]

END OF TEST